Attachments:

| Manufacturer |  | Middle Bolt | Sealing Surface | Max. Operating Pressure, MPa | Gasket Material | Glue | Quality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Durango | YLAA | N | Flat | 3.1 | Special Neoprene: Garlock5280 | LOTITE 380 | In-plant FPY 100% for YLAA/YLPA, field failure rate about 60% for YLPA only; YLAA unknown |
|  | YLPA |  |  | 4.5 |  |  |  |
| Carrier/McQuay |  | Y | Flat | 2.5~4.2 | Non-asbestos | N | Unknown |
| Wuxi | YAEP YEAS YCAB | Y | Grooved or Flat | 2.5~3.1 | Common Neoprene or Non-asbestos (Klinger6327) | N | Overall: In-plant FPY 94%; YLAA: FPY 90.2%. Field failure rate zero; |
|  | YLAA | N | Flat | 3.1 | Non-asbestos : Klinger6327 | N |  |
|  | YCAE | Y | Female | 4.2 |  |  |  |
|  | YLPA | Y | Flat | 4.5 | Non-asbestos : AFM34 | N |  |
|  | YSPA | N | Female | 2.1 | Common Neoprene | LOTITE 380 | In-plant FPY 100%; Field failure rate zero |
| Note: - Flat and female have similar machining cost, but the grooved is much more expensive; - In China, Garlock 5280 has similar cost with Klinger6327, but with five times of common neoprene; AFM 34 has seven times of common neoprene. |  |  |  |  |  |  |  |